# Binary Decompression Quick Start

A detailed instruction is currently being written.

Python must be installed to run these scripts. It is available in the Software Center.

There is more than one binary format:

Eg AWS data is binary format 001; Triaxys binary is 127; Motus is 128.

The formats are laid out in csv files call ‘bf-###.csv’, where ### is the 3-digit number of the format (lowest value is 001, highest is 254).

The DataformatDecoder.py program will decode any of these as long as the format file is in the same folder as the program.

There are two ways to decode binary data.

One message at a time…

- Put the binary file in the same folder as the DataformatDecoder.py program and rename it ‘dataoutput.sbd’

- Right click DataformatDecoder.py and select Edit with Idle.

- Press F5 to run the program.

- The script will run and display the output in a Python shell window

Multiple messages…

- Create a subfolder called ‘sbddatafolder’.

- Put all the sbd files into this folder. Only the files that have the same IMEI number as the alphabetically first file will be processed.

- Double click SBDDataCompiler.py.

- The script will run and put all the data in an appropriately named and time-stamped csv file in the sbddatafolder.